Work completed

____________________________________________________________________________

BFS is working( Could return to start even after traversing 50 tiles).

Navigation is fixed.

- Remembers black tiles

Camera could now detect cognitive targets and letter targets.

Work that needs to be done

____________________________________________________________________________

Wire the second robot.

Test climbing navigation

Memory saving hasn’t been implemented.

Obstacle avoidance.

Issues:

____________________________________________________________________________

Power issues on the first robot

- Very faint PWM whining.

- Robot stops abruptly/ completely loses control, sensors start blinking.

- Suspected current dips.

OpenMV find_circles() misidentifies letters as circles, sending garbage data.

- Find circles is also highly unstable, dependant on lighting conditions.

Arduino sometimes fail to respond to camera detections.

- Used preliminary blob detection(faster) to tell the robot to stop sooner.

- However, theres the issue of the Arduino misidentifying blobs.

Motors 2 and 3 are slower on the new robot

- Rigorous variable-controlled testing required to diagnose issue.

Dispensing is  inaccurate on the first robot.

- The stepper does not return to its original position after dispensing

Confusion between black and blue tiles

- Can be fixed by tuning, but it is strange that the color sensor records different values.