Sprint #2 Continued Development

Goals:

- LONG TERM: Complete secondary robot, including mounting and development, with functioning mechanical components and sensors, with a focus on scoring and victim detection.

- SHORT TERM

- Wire management, develop the new robot with significantly better wire management than its predecessor

- Stepper motor control and block scoring( Baichen Luo).

- 3d Printing and assembly

- Silicone wheel casting and mold design

- Obstacle avoidance(Baichen Luo).

Documentation Research, and Development (Leo)

Mounted the suspension system for the new robot

Silicone wheels:

New wheels were designed, cast and printed(Leo)
ELECTRICAL COMPONENT OF THE ROBOT (LEO)

Goals

Make the robot's wiring more reliable and improve how the sensors work by using better wiring and adding lights to the sensors.

Wire Management and Reliability Improvements

What I Did: Replaced (SOME) solid core wires with stranded wires to make the robot more durable.

Why This Matters:

Solid core wire breaks easily at the solder joints when the robot moves and vibrates. Stranded wire bends better and doesn't break as easily. This makes the connections much less likely to fail during the competition

Removed (desoldered) the old solid core wires from the robot. Replaced them with stranded wire that's the right thickness for each sensor. Organized (some) of the wires neatly to avoid putting stress on the connections. Made sure (SOME) solder joints were secure and wouldn't break – Work in progress still

Results:

Stronger connections that won't break from movement/ unplug during competitions. Much lower chance of wires failing during competition. Slightly Cleaner and easier to work with wiring

LED Lighting for Sensors

What I Did: Added white LEDs next to all the sensors so they always have the same lighting.

The Problem: Room lighting changes between our workspace and the competition venue. When lighting changes, the sensors give different readings and need to be recalibrated. Color sensors and distance sensors are especially affected by lighting changes

Solution: Installed white LEDs next to all sensors (OpenMV CAMERA, color sensors). The LEDs give the sensors their own consistent light no matter what the room lighting is like Placed the LEDs carefully to avoid glare while making sure the sensors are lit evenly

Why This Helps:  Removed the need for calibration and overcomplicating the code that barely works further. Sensors work the same everywhere. More reliable: Fewer bad readings from the sensors. Saves time: Don't need to spend time adjusting sensors before competition. Repeatable performance: Robot behaves the same way no matter where we run it

Overall Impact

These improvements help make the new robot more reliable. By fixing the wiring problems and ensuring the sensors always work consistently, the robot's electrical component now has a solid foundation for the rest of the robot to work properly during competition.

Obstacle avoidance(Baichen):

Idealization:

Architecture: I did not want to make a bloated function with lots of if loops inside, as it would be messy. Instead, I decided to opt for a state machine architecture.

https://junior.robocup.org/wp-content/uploads/2026/01/RCJRescueMaze2026-draft.pdf

My State machine has 3 states: Turn, Parallel, and backward. The robot first uses its front distance sensors to detect the obstacle( detecting is labelled in green). Based on whether the left or right sensor is blocked, the robot would choose to turn left or right until the blocked sensor is blocked. Next, the robot uses PID to go forward while paralleling, in the goal of edging towards the wall. When the robot is too close to the wall, the robot would go backward until it enough far away from the wall. States 2 and 3 keep repeating until the sensors report the robot is parallel to the wall, and can proceed forward.

( This was inspired by parking).

Product:

void obstacleavoidance(int leftright){ // leftright determines to manuver left or right.

while(true){

switch (steps){

case TURN:{

if(leftright == 1){ // obstacle at left

while(measure(7) < MIN_DIST){

motorB->run(BACKWARD);

motorD->run(BACKWARD);

motorA->setSpeed(255);

motorB->setSpeed(255);

motorC->setSpeed(255);

motorD->setSpeed(255);

}

}

else if(leftright == 0){ // obstacle at right

while(measure(1)<MIN_DIST){

motorA->run(BACKWARD);

motorC->run(BACKWARD);

motorA->setSpeed(255);

motorB->setSpeed(255);

motorC->setSpeed(255);

motorD->setSpeed(255);

}

}

fullstop();

delay(200);

motorA->setSpeed(255);

motorB->setSpeed(255);

motorC->setSpeed(255);

motorD->setSpeed(255);

delay(300);

fullstop();

delay(200);

steps = PARALLEL;

break;

}

case PARALLEL:{

PID pid(1,0,0.1);

if(leftright == 1){

while(measure(2)>=25){

double increment = pid.getPID(abs(measure(3)-measure(2))); // get close to the edge as possible.

motorA->setSpeed(constrain(100-increment,30,255));

motorB->setSpeed(constrain(100+increment,30,225));

motorC->setSpeed(constrain(100-increment,30,255));

motorD->setSpeed(constrain(100+increment,30,225));

if(abs(measure(3)-measure(2))<=5){

fullstop();

delay(200);

steps = FWD;

goto end;

}

}

}

else if(leftright == 0){

while(measure(6)>=25){

double increment = pid.getPID(abs(measure(6)-measure(5))); // get close to the edge as possible.

motorA->setSpeed(constrain(100-increment,30,255));

motorB->setSpeed(constrain(100+increment,30,225));

motorC->setSpeed(constrain(100-increment,30,225));

motorD->setSpeed(constrain(100+increment,30,225));

if(abs(measure(6)-measure(5))<=5){

fullstop();

delay(200);

steps = FWD;

goto end;

}

}

}

steps = BACKTRACK; // put switch step in front of end( always meet it)

break;

end:

break;

}

case BACKTRACK:{

Serial.println("backtracking");

if(leftright == 0){

while(measure(6)<=25){

motorA->run(BACKWARD);

motorB->run(BACKWARD);

motorC->run(BACKWARD);

motorD->run(BACKWARD);

motorA->setSpeed(100);

motorB->setSpeed(100);

motorC->setSpeed(100);

motorD->setSpeed(100);

}

}

else if(leftright == 1){

while(measure(2)<=40){

motorA->run(BACKWARD);

motorB->run(BACKWARD);

motorC->run(BACKWARD);

motorD->run(BACKWARD);

motorA->setSpeed(100);

motorB->setSpeed(100);

motorC->setSpeed(100);

motorD->setSpeed(100);

}

}

fullstop();

delay(200);

steps = PARALLEL;

break;

}

case FWD:{

fwd(300);

}

}

}

}

Reflection:

https://www.adafruit.com/product/3317?srsltid=AfmBOorYx9HujPoq666eJgHWlfeGMB_g-yDZlW-NYIVmW3EhVjr3tpxh

The code is logically correct, but still requires vast amounts of tuning for it to work. Firstly, the Vl53L0X distance sensor has a minimum range of 30mm, which is inconvenient when dealing with distances less than that when avoiding obstacles. Secondly, the PID paralleling needs to be tuned, which would take a lot of time. The third issue( perhaps the most important of them all), is the problem that the wheels are wider than the distance sensors. This means that when the distance sensor things the robot is clear of the obstacle, the wheels are still going to bump into it. Therefore, we made the wheels less wide for the next robot to avoid this issue.

Servo dispenser(Baichen):

Idealization:

The 28BYJ-48 is a 5V stepper, and the robot is powered by a 12v power source. Therefore, I need a voltage buck to convert 12v to 5v to power the stepper.

https://www.pololu.com/product/2851

Wiring diagram:

Product:

I made a dispenser class for the dispenser.

Dispenser.h:

#ifndef dispenser_h

#define dispenser_h

class dispenser{

public:

int leftDispensed; int rightDispensed;

int incr, offset, steps_per_rev;

dispenser(int, int, int);

void rotate(int);

void dispenseLeft(char);

void dispenseRight(char);

};

using sdispenser = dispenser; // Warning! this might not work on somebody else's computer!!!

#endif

Dispenser.cpp:

#include <Arduino.h>

#include "dispenser.h"

#include <Stepper.h>

extern Stepper myStepper;

extern int medkits;

dispenser::dispenser(int _incr, int _offset,int _steps_per_rev){

incr = _incr;

offset = _offset;

steps_per_rev = _steps_per_rev;

leftDispensed = 0; rightDispensed = 0;

}

void dispenser::rotate(int degrees){

myStepper.setSpeed(7);

long steps = ((long)degrees*steps_per_rev)/360;

myStepper.step(steps);

}

void dispenser::dispenseLeft(char victim){ // clockwise

switch(victim){

case 'H':

rotate(-(incr*2+offset+leftDispensed*incr));

leftDispensed += 2;

medkits -= 2;

rotate(offset+leftDispensed*incr);

break;

case 'S':

rotate(-(incr*1+offset+leftDispensed*incr));

leftDispensed += 1;

medkits -= 1;

rotate(offset+leftDispensed*incr);

break;

case 'U':

break;

}

}

void dispenser::dispenseRight(char victim){ // clockwise

switch(victim){

case 'H':

rotate(incr*2+offset+rightDispensed*incr-11);

rightDispensed += 2;

medkits -= 2;

rotate(-(offset+rightDispensed*incr-11));

break;

case 'S':

rotate(incr*1+offset+rightDispensed*incr-11);

rightDispensed += 1;

medkits -= 1;

rotate(-(offset+rightDispensed*incr+11));

break;

case 'U':

break;

}

}

Reflection:

The Servo worked quite well, but there were a few issues. Firstly, I renamed the dispenser class to sdispenser for alphabetical reasons(Arduino IDE compiles files by alphabetical order). However, there was actually no such issue, so I switched it back. Unfortunately, the IDE thinks the sdispenser file is still there, which is not possible. Even after closing Arduino IDE and restarting the computer, the error still occurred. Therefore, I created a sdispenser.h and referenced it, and the error went away somehow. This might not work on somebody else’s computer.

Due to the servo being badly mounted, there is some “slack” before meeting the resistance of the motor, making dispensing inaccurate.  Therefore, I added an offset( based on the angular size of the slack region) to compensate.

Christopher:

# Navigation and Logic Improvements

## 1. Blue Tile Avoidance Strategy

According to competition rules, blue tiles represent difficult terrain and impose movement penalties (e.g., mandatory stop) . Re-visiting such tiles unnecessarily reduces efficiency and increases the risk of lack of progress (LoP).

To address this, the navigation system was extended with state-aware tile classification:

- A new attribute was added to the MazeTile class:

  - isBlue

  - visitedBlue

### Initial Implementation

- When a tile is detected as blue:

  - It is marked in the map.

  - The navigation algorithm avoids revisiting it during BFS/path planning.

### Issue Encountered

- The robot could become trapped when:

  - The only available path required passing through a previously visited blue tile.

### Final Solution

A conditional override was introduced:

- Blue tiles are treated as high-cost nodes instead of forbidden nodes.

- Logic:

  - Avoid revisiting blue tiles if alternative paths exist.

  - Allow revisiting only when no other valid path is available.

### Result

- Maintains rule compliance and efficiency.

- Prevents deadlocks in constrained maze configurations.

- Improves robustness of exploration.

## 2. Botched Turn Recovery

During testing, inconsistent turning behavior was observed:

- The robot occasionally stops at incorrect angles (e.g., ~45° instead of 90°).

- Likely causes:

  - Motor power fluctuations

  - Surface inconsistencies

  - IMU drift

### Implementation

A new state was introduced in the robot state machine:

- BOTCHED_TURN

### Detection

The system detects a failed turn when:

- The robot's heading is not close to a cardinal direction after a turn.

- Uses headingToCardinal() to evaluate deviation.

### Correction Strategy

When triggered:

- Determine the nearest cardinal direction.

- Rotate to correct orientation.

- Resume normal execution (EXECUTE_MOVE state).

### Design Considerations

- Does not reset navigation state (position, direction, map remain consistent).

- Ensures continuity in BFS and exploration logic.

### Result

- Eliminates accumulated orientation errors.

- Improves reliability of long navigation sequences.

- Reduces cascading navigation failures.

## 3. Navigation Logic Completion

The navigation system was refined to ensure full autonomy and stability:

### Key Improvements

- Fixed logical inconsistencies in movement decision flow.

- Ensured correct priority ordering:

  - Unvisited + open tiles

  - Visited but valid tiles

  - Backtracking (BFS)

### Integration with Tile States

- Tile attributes now include:

  - Walls

  - Visited state

  - Special tiles (black, blue, etc.)

### Behavior Enhancements

- Avoids black tiles (treated as obstacles per rules).

- Handles blue tiles with conditional revisiting logic.

- Maintains consistent map updates for BFS.

### Result

- Stable exploration without loops or stalls.

- Better handling of edge cases (dead ends, constrained paths).

- Fully functional autonomous navigation aligned with competition requirements.